Introduction to Electrical Engineering

Lab Report

Name: Pranesh Chowdhury

ID: 202003112

Course ID: 112

Section: 2

Experiment No: 6

Experiment Name: Verification of Reciprocity Theorem

Objective:

In this experiment, we are going to verify the reciprocity theorem. The reciprocity theorem works by the current 'I' in any branch of a network, due to a single voltage source 'E' anywhere else in the network, will equal the current through the branch in which the source was originally located if the source is placed in the branch in which the current 'I' was originally measured.

List of Equipment:

- Wires

- Resistors

- PSIM Software

- DC Power Supply

- Ammeters

- Voltmeters

Circuit Diagram:

Circuit 1:

Here,

Rt = R1 + {R2 || (R3+R4)}

- 82 + {100 || (68+47)} ohms

- 135.49 ohms

Is = E / Rt

- 5V / 135.49 ohms

- 0.036903535 A

I = {(100 * 0.036903535) / (100+68+47)} A

- 0.017164435 A

Circuit 2:

Here,

Rt = (R1 || R2) +R3+R4

- {(82 || 100) +68+47} ohms

- 160.0549451 ohms

Is= E / Rt

- 5V/160.0549451 A

- 0.031239272 A

I = {100/ (100+82)} ohms/0.031239272 A

- 0.017164435 A

From Circuit 1 & 2 we can see that both current values are same. So, the reciprocity theorem is satisfied here.

Circuit 3:

Here,

Rt = {R1 (R3+R4) || R3}

- 135.488372 Ohms

Is = E / Rt

- 5 / 135.488372 A

- 0.0369034 A

I2 = (115 + 0.0369034) / (115 + 100)

- 0.01973881 A

Circuit 4:

Here,

Rt = [{R1 || (R4 + R3)} + R2]

- 147.8680203 Ohms

Is = E / Rt

- 5 / 147.8680203 A

- 0.0338139 A

I = (115 * 0.0338139) / (115 + 82)

- 0.019739 A

From Circuit 3 & 4 we can see that both current values are same. So, the reciprocity theorem is satisfied here.

Discussion:

We have to be taken care whereas setting batteries and ammeter since able to get default values. We evaluate the four circuits using PSIM software. I faced some troubles in this portion by getting default values since of relocation of ammeter and battery.